URBAN INEQUALITY IN VADODARA

Geo-Spatial Analysis of Opportunity, Risk & Accessibility

M.Sc. Statistics — Definitive Technical Summary

Vadodara Municipal Corporation · 32 Wards · 2024–2026

github.com/threed2y/URBAN-INEQUALITY-IN-BARODA

# 1. Project Overview

This thesis constructs a ward-level Urban Opportunity Index (UOI) for Vadodara, India, combining four domains of spatial inequality — healthcare accessibility, educational access, public transit coverage, and flood exposure risk — into a single composite measure validated by Bayesian spatial modelling, SWMM hydrodynamic simulation, and Graph Convolutional Network–BYM2 hybrid inference.

Research question How does spatial inequality in urban opportunity manifest across Vadodara's 32 wards, and which neighbourhoods bear a compounded burden of poor accessibility and high flood risk?

## 1.1 Scope and Objectives

- Construct and validate a ward-level Urban Opportunity Index (UOI) with four normalised components

- Quantify spatial autocorrelation via global Moran's I and LISA cluster maps

- Fit a BYM2 Bayesian hierarchical spatial model in PyMC 5 with proper ICAR priors

- Simulate a 4-hour cloudburst using the SWMM DYNWAVE engine and map flood-induced road failure

- Build a GCN + BYM2 hybrid model for graph-informed spatial inequality prediction

- Provide policy-actionable ward typologies — Opportunity × Flood Risk quadrant classification

## 1.2 Data Sources

| Source | Data | Resolution | Access |
| --- | --- | --- | --- |
| OpenStreetMap (OSMnx) | Road network, hospitals, schools, bus stops | Edge-level | Free |
| Open Vadodara Smart City Bus | GTFS transit schedules and stop locations | Stop-level | Free |
| SRTM 30m DEM | Terrain elevation for SWMM hydraulics | 30m raster | Free (NASA) |
| VMC Ward Boundaries | 32 ward administrative polygons | Ward-level | Free |
| OSM Overture / city2graph | Heterogeneous spatial graph construction | Node/edge | Free |

# 2. Urban Opportunity Index — Construction

The UOI is a min-max normalised composite on the scale 0–100 (higher = better), built from four independently computed sub-scores. The baseline weighting reflects the policy priorities identified in the Vadodara Smart City Mission.

## 2.1 Component Definitions

| Component | Weight | Metric | Script | Direction |
| --- | --- | --- | --- | --- |
| Healthcare (H) | 30% | Drive-time to nearest hospital / clinic (OSMnx, network isochrone) | 03_accessibility_engine.py | Higher access = higher score |
| Education (E) | 30% | Walk-time to nearest school / college (OSMnx) | 03_accessibility_engine.py | Higher access = higher score |
| Transit (T) | 20% | Bus stop density + route coverage (GTFS via city2graph) | 03b_transit_accessibility.py | Higher density = higher score |
| Flood Risk (F) | 20% | % ward area in flood exposure zone (SWMM post-storm) | 04_risk_assessment.py | Lower exposure = higher score |

## 2.2 UOI Formula

Each component xi is min-max normalised to [0,1] across all 32 wards. The flood risk component is inverted (1 − F̂) so that high flood exposure reduces the score.

UOI_i = 0.30·Ĥ_i + 0.30·Ê_i + 0.20·T̂_i + 0.20·(1 − F̂_i)

The resulting composite score is rescaled to 0–100 for interpretability. Kolm–Pollak EDE adjustment is applied with aversion parameter κ swept from 0.1 to 2.0 to test inequality-adjusted rankings.

## 2.3 Sensitivity Analysis Results

A six-step sensitivity protocol was applied: OAT (±5pp weight perturbation), Monte Carlo with Dirichlet draws (N=10,000), Sobol' first-order indices, Kolm–Pollak κ-sweep, and spatial CI mapping.

| Component | OAT Mean Rank Shift | MC 95% CI Width (W0) | Sobol' S₁ | Rank Stability ρ |
| --- | --- | --- | --- | --- |
| Healthcare | ±1.44 | ±17.5 | 0.34 | 0.977 |
| Education | ±1.41 | ±16.8 | 0.32 | 0.974 |
| Transit | ±1.16 | ±12.4 | 0.19 | 0.981 |
| Flood Risk | ±1.31 | ±14.2 | 0.27 | 0.979 |

Key sensitivity finding Healthcare and Education are the highest-sensitivity components (mean rank shift ~1.4). 9/32 wards achieve classification robustness (≥80% quintile agreement across all MC draws). The EDE Spearman ρ remains ≥0.99 across the full κ range — the inequality-adjusted ranking is stable.

# 3. Spatial Statistics

## 3.1 Moran's I and LISA

Global Moran's I tests for spatial autocorrelation in UOI under the Queen contiguity weight matrix (built by city2graph.contiguity_graph). A significant positive Moran's I indicates that high-opportunity wards cluster together — a necessary condition justifying the BYM2 spatial random effect.

I = (n / S₀) · (Σᵢ Σⱼ wᵢⱼ (xᵢ − x̄)(xⱼ − x̄)) / (Σᵢ (xᵢ − x̄)²)

LISA decomposition (Anselin 1995) identifies four ward typologies: High-High (spatial clusters of opportunity), Low-Low (deprivation clusters), High-Low and Low-High (spatial outliers). This map directly informs the intervention priority ranking.

## 3.2 Spatial Regression Model Hierarchy

Following Anselin (1988), the Lagrange Multiplier tests on OLS residuals determine whether a Spatial Lag Model (SLM) or Spatial Error Model (SEM) is appropriate before escalating to BYM2.

| Model | When to use | Coefficient | Interpretation |
| --- | --- | --- | --- |
| OLS | Baseline; LM tests non-significant | β | Global covariate effect (biased if spatial autocorrelation exists) |
| SLM (SAR) | LM-Lag significant, RLM-Lag > RLM-Error | ρ | Neighbouring ward UOI influences local UOI — spillover effect |
| SEM | LM-Error significant, RLM-Error > RLM-Lag | λ | Residual spatial clustering from unmeasured confounders |
| GWR | After global model; spatially varying β | β(u,i) | Local coefficient maps — does flood risk suppress opportunity uniformly? |
| BYM2 (Bayesian) | Primary model; captures both structured and unstructured spatial variation | φ, σ_b | Posterior probability of spatial clustering; 95% credible intervals per ward |

# 4. BYM2 Bayesian Spatial Model

## 4.1 Model Specification

The Besag-York-Mollié 2 model (Riebler et al. 2016) is the primary inferential model. It decomposes the ward-level spatial random effect into structured (ICAR) and unstructured (IID) components with a mixing parameter φ that directly measures the proportion of spatial variance attributable to spatial clustering.

BYM2 specification y_i = intercept + β·x_i + b_i + ε_i, where b_i = σ_b·(√φ·u*_i + √(1−φ)·v*_i). The ICAR component u* captures spatial clustering, v* captures ward-specific noise, σ_b is the marginal standard deviation, and φ ∈ [0,1] measures spatial fraction.

## 4.2 Mathematical Specification

y_i | μ_i, σ_ε ~ Normal(μ_i, σ_ε²)

μ_i = α + Σⱼ βⱼ xᵢⱼ + bᵢ

bᵢ = σ_b · (√φ · u*ᵢ + √(1−φ) · v*ᵢ)

u* ~ ICAR(W)   [scaled, sum-to-zero constraint]

v*ᵢ ~ Normal(0, 1)   [IID unstructured]

Penalised Complexity (PC) priors are placed on the hyperparameters: P(σ_b > 1.0) = 0.01 and φ ~ Beta(0.5, 0.5). The ICAR precision matrix Q = D − W is rank-deficient by 1; the scaling factor is computed via eigendecomposition to make σ_b interpretable as a marginal standard deviation.

## 4.3 Implementation in PyMC 5

The standard pm.CAR implementation is NOT used — it approximates ICAR with α=0.99 and hardcoded τ, producing biased σ_b estimates. The correct implementation uses an explicit ICAR log-potential:

ICAR log-prob ∝ −(1/2) Σ_(i~j) (u_i − u_j)²

Adjacency arrays (node1, node2) are generated by city2graph.contiguity_graph with Queen contiguity and extracted from the MultiIndex edge GeoDataFrame. The MCMC configuration uses NUTS with target_accept=0.90 and 4 chains × 2000 draws.

| Parameter | Prior | Interpretation | Posterior (expected) |
| --- | --- | --- | --- |
| α (intercept) | Normal(0, 1) | Baseline UOI | city mean UOI |
| β_k (fixed effects) | Normal(0, 1) | Component k coefficient | positive for access metrics |
| σ_b | HalfNormal(0.5) [PC] | Marginal spatial SD | ~0.3–0.8 for ward-level data |
| φ | Beta(0.5, 0.5) [PC] | Spatial fraction of σ_b | >0.5 → spatially structured |
| σ_ε | HalfNormal(1.0) | Residual noise SD | smaller than σ_b if model fits well |

# 5. SWMM Hydrodynamic Flood Simulation

## 5.1 Network Construction (build_swm_fixed.py)

The SWMM .inp file is built from the Vadodara major road network (primary + secondary + tertiary roads only, ~700–900 nodes) filtered from OSMnx using custom_filter to avoid the 18,000-node full drive network that causes DYNWAVE to run for 17+ minutes.

Critical: Network size Full 'drive' network: ~18,000 nodes → 17+ minute runtime (DYNWAVE substepping to <1ms). Major roads only: ~700 nodes → <30 second runtime. The filter is: ["highway"~"primary¦secondary¦tertiary¦trunk"]

## 5.2 Root Causes of the Original 17-Minute Hang

| Cause | Original value | Fixed value | Effect |
| --- | --- | --- | --- |
| Network scale | All drive roads (~18k nodes) | Major roads only (~700 nodes) | 20× compute reduction |
| Inflow per junction | 0.05 CMS constant (900 CMS total) | Storm TIMESERIES, peak 0.005 CMS/node | 100× inflow reduction |
| Elevations | np.random.uniform(30, 40) — 50% uphill conduits | Synthetic DEM slope toward Vishwamitri river | Eliminates closed uphill loops |
| MaxDepth | 0 (no junction storage) | 2.0m, Aponded=200m² | Prevents instant surcharge cascade |
| Outfalls | 1 dummy outfall (bottleneck) | 5 outfalls at lowest-elevation nodes | Reduces backpressure |

## 5.3 Storm Hyetograph

A triangular design storm is used, representative of Vadodara's 10-year return period rainfall (~50mm/hr peak). Peak flow per node is 0.005 CMS, representing a 500m² catchment area per junction with 0.70 runoff coefficient.

Q_peak = (50/3600 m/s) × 500 m² × 0.70 = 0.0049 ≈ 0.005 CMS

The storm timeline has 8 steps from dry baseline through 1h30m peak (intensity=1.0) to 4h recession. DYNWAVE routing step is 5 seconds, satisfying the CFL condition for the shortest major road segment (~50m at wave speed ~3.1 m/s: CFL dt ≤ 50/3.1 = 16s).

## 5.4 Ward-Level Flood Aggregation

SWMM output (edge-level max depth) is joined back to the OSMnx major-road GeoDataFrame by (u, v, key) index, then spatially joined to ward polygons. Three ward-level flood metrics are computed and written to vadodara_final_uoi_balanced.gpkg:

- swmm_flood_road_pct — percentage of road length with max depth exceeding road-type threshold

- swmm_mean_depth_m — mean maximum flood depth across ward roads

- swmm_max_depth_m — worst-case flood depth encountered in the ward

Road-type-specific thresholds are applied: arterial=0.50m, primary=0.45m, secondary=0.35m, tertiary=0.30m, residential=0.25m. The SWMM flood rate feeds directly into GNN.py as a dynamic replacement for the static flood_exposure_pct raster layer.

# 6. city2graph Integration

## 6.1 Library Role

city2graph (Sato 2025, doi:10.5281/zenodo.15858845) is a Python library that converts geospatial datasets into graph representations with integrated interfaces for GeoPandas, NetworkX, and PyTorch Geometric. It replaces manual libpysal adjacency construction throughout the pipeline.

## 6.2 Functions Used

| Function | Purpose | Output | Used in |
| --- | --- | --- | --- |
| c2g.contiguity_graph(wards, 'queen') | Ward adjacency for BYM2 | MultiIndex edge GeoDataFrame (src_idx, tgt_idx) | 10_city2graph_integration.py |
| c2g.group_nodes(wards, pois) | POI → ward bipartite edges | nodes_dict, edges_dict | 10_city2graph_integration.py |
| c2g.knn_graph(centroids, k=3) | KNN spatial proximity graph | nodes GDF, edges GDF | Robustness checks |
| c2g.delaunay_graph(centroids) | Delaunay triangulation | nodes GDF, edges GDF | Robustness checks |
| c2g.gdf_to_nx(nodes, edges) | Convert to NetworkX graph | nx.Graph | GraphML export |
| c2g.gdf_to_pyg(nodes, edges) | Convert to PyTorch Geometric | HeteroData | GNN training |
| c2g.load_gtfs(path) | Parse GTFS transit feed | DuckDB connection | 03b_transit_accessibility.py |
| c2g.travel_summary_graph(con) | Stop-to-stop transit graph | nodes GDF, edges GDF | Transit accessibility |

BYM2 adjacency extraction city2graph edge MultiIndex: edges_gdf.reset_index() → columns ['level_0','level_1']. These are 0-indexed ward positions directly usable as BYM2 node1/node2 arrays. No mapping needed if wards are sorted by ward_id before calling contiguity_graph.

# 7. GNN + BYM2 Hybrid Model

## 7.1 Architecture

The hybrid model uses a 3-layer Graph Convolutional Network (GCN) to learn spatially-contextualised ward embeddings from the road network topology, then feeds these as covariates into the BYM2 Bayesian model. This is justified because GNNs can capture both the topology of city networks and spatial context — naturally suited for predicting how connectivity patterns drive inequality.

## 7.2 Critical Bugs Fixed

| Bug | Original code | Correct implementation | Statistical consequence |
| --- | --- | --- | --- |
| Wrong spatial prior | pm.CAR(alpha=0.99, tau=1.0) | ICAR Potential: pm.Potential('icar', icar_logp(u_raw, node1, node2)) | CAR ≠ ICAR; biased σ_b and φ; wrong credible intervals |
| GNN predictions as fixed covariate | gnn_pred.detach().numpy() → fixed array | LOO-CV predictions OR use penultimate embeddings as features | Treats GNN uncertainty as zero; attenuation bias in β_gnn |
| Informative prior on β_gnn | Normal(mu=1, sigma=0.2) | Normal(0, 1) | Forces model to trust GNN even if miscalibrated |
| Over-aggressive MCMC target | target_accept=0.99 | target_accept=0.90 | Extreme slowness; effective sample size collapses |
| No spatial cross-validation | Train on all 32 wards | LOO-CV: train on 31, predict held-out | Double-dipping: in-sample GNN predictions inflate confidence |

## 7.3 Corrected BYM2 Specification

y_i = α + β_gnn·ŷ_GNN_i^(LOO) + σ_b·(√φ·u*_i + √(1−φ)·v*_i) + ε_i

ŷ_GNN^(LOO) are out-of-sample GNN predictions from leave-one-out spatial cross-validation. With n=32, this is feasible: 32 fits of a GCN on 31 wards. The posterior of β_gnn serves as a diagnostic — if it concentrates near 0, the GNN is not adding value beyond the spatial random effect.

# 8. Reproduction Steps — Clean Pipeline

## 8.1 Environment Setup

Python 3.11+ is required. The venv name is arbitrary; the project uses LA-311 in the provided terminal output.

# Create and activate virtual environment

python -m venv venv && source venv/bin/activate

# Core pipeline
pip install geopandas osmnx libpysal esda pymc pytensor arviz
pip install city2graph pyswmm streamlit pydeck scipy tqdm

# GNN support
pip install "city2graph[cpu]" torch torch_geometric

# npm for pptxgenjs / docx (optional - for report generation)
npm install -g docx pptxgenjs

## 8.2 Execution Order

| Step | Script | Input | Output | Note |
| --- | --- | --- | --- | --- |
| 1 | 03_accessibility_engine.py | OSM (live download) | wards_accessibility.gpkg | Drive/walk isochrones to hospitals, schools |
| 2 | 03b_transit_accessibility.py | data/raw/vadodara_gtfs.zip | wards_transit.gpkg | Requires Open Vadodara GTFS zip |
| 3 | 04_risk_assessment.py | SRTM DEM raster | wards_flood.gpkg | Place SRTM .tif in data/raw/ |
| 4 | 05_calculate_uoi.py | wards_*.gpkg files | vadodara_final_uoi_balanced.gpkg | UOI composite 0–100 |
| 5 | 05b_prepare_bym2_inputs.py | vadodara_final_uoi_balanced.gpkg | bym2_inputs.npz | Standardises features, builds adjacency |
| 6 | 10_city2graph_integration.py | vadodara_final_uoi_balanced.gpkg | results/city2graph/* | city2graph graphs + BYM2 node arrays |
| 7 | 06_bym2_pymc_model.py | bym2_inputs.npz + bym2_node1/2.npy | wards_bym2_results.gpkg + bym2_idata.nc | Primary spatial model |
| 8 | build_swm_fixed.py | OSM (live download) | vadodara_overland.inp | Major roads only, storm TS |
| 9 | run_swmm_fixed.py | vadodara_overland.inp | swmm_road_failures.csv + updated .gpkg | Adds swmm_flood_road_pct |
| 10 | GNN.py | vadodara_final_uoi_balanced.gpkg (gnn_bym2_results layer) | gnn_bym2_results layer in .gpkg | Fix bugs before running (see §7.2) |
| 11 | dashboard.py | vadodara_final_uoi_balanced.gpkg | Streamlit app (port 8501) | streamlit run scripts/Python/dashboard.py |

## 8.3 Known Data Requirements

- GTFS zip: download from open.vadodara.gov.in → Smart City Bus → place at data/raw/vadodara_gtfs.zip

- SRTM DEM: download SRTM 1-arc-second tile N22E073 from earthexplorer.usgs.gov → data/raw/srtm_n22e073.tif

- Ward shapefile: from VMC or synthetic grid in 05b_prepare_bym2_inputs.py (32 synthetic wards used for testing)

- Internet connection required for Steps 1, 2, 8, 9 (OSMnx downloads)

# 9. Complete File Inventory

| File | Type | Purpose | Status |
| --- | --- | --- | --- |
| uoi_sensitivity_analysis.py | Python | OAT + Monte Carlo + Sobol + Kolm-Pollak sensitivity | Complete |
| 05b_prepare_bym2_inputs.py | Python | BYM2 data preparation bridge | Complete |
| 06_bym2_pymc_model.py | Python | BYM2 ICAR model in PyMC 5 | Complete |
| 10_city2graph_integration.py | Python | city2graph contiguity/proximity/POI graphs | Complete |
| build_swm_fixed.py | Python | SWMM .inp builder (major roads, storm TS) | Complete |
| run_swmm_fixed.py | Python | SWMM runner + ward aggregation | Complete |
| GNN.py | Python | GCN + BYM2 hybrid (bugs documented in §7.2) | Needs fixes |
| dashboard.py | Python | Streamlit 5-KPI dark-theme dashboard | Complete |
| vadodara_network.html | HTML | Self-contained flood network visualisation | Complete |
| vadodara_overland_fixed.inp | SWMM | Fixed .inp (5 missing junctions patched) | Complete |
| UOI_Bayesian_SpatioTemporal_Proposal.docx | Word | Spatio-temporal extension proposal | Complete |
| c2g_graph_overview.png | PNG | 4-panel city2graph contiguity + POI visualisation | Complete |
| bym2_prior_predictive_check.png | PNG | Prior predictive check figure | Complete |

# 10. Theory Extracts for Thesis Citation

## 10.1 Composite Index Theory

The OCCIM (Optimum Combinational Composite Index Model) approach integrates GIS with synthetic indices to measure spatial variation in urban conditions. A composite index for urban household living conditions is computed based on three inter-connected dimensions — housing conditions, amenities level, and asset possession — providing insight into the geographic distribution considering spatial patterns, hot spots, and cluster identification.

Spatial heterogeneity is measured using Moran's I at the global level and Getis-Ord Gi* to identify influential locations through clusters and hot spot detection. Model consistency is assessed using the R-squared value of regression analysis. The UOI generalises this framework by incorporating accessibility metrics and normalising to a 0–100 scale with explicit inequality adjustment via the Kolm–Pollak EDE.

## 10.2 Spatial Regression Theory

Spatial econometrics addresses the failure of OLS when observations are spatially autocorrelated. Residual spatial autocorrelation breaks the independence assumption, inflating Type I error rates and producing biased, inefficient coefficient estimates. The Anselin (1988) Lagrange Multiplier tests provide a data-driven procedure to select between the Spatial Lag Model and Spatial Error Model.

The Spatial Lag Model incorporates the spatially-lagged dependent variable as an explanatory variable: y = ρWy + Xβ + ε. The coefficient ρ ∈ (−1, +1) quantifies spatial spillover — neighbouring ward UOI influences local UOI. This is appropriate when there are genuine neighbourhood externalities, such as a major hospital corridor benefiting multiple adjacent wards.

The Spatial Error Model absorbs unmeasured spatially-clustered confounders into the error structure: y = Xβ + u, u = λWu + ε. λ ∈ (−0.99, 0.99) — a positive λ indicates residual spatial clustering from omitted variables such as historical settlement patterns or socioeconomic segregation.

## 10.3 BYM2 Penalised Complexity Priors

The BYM2 reparameterisation (Riebler et al. 2016) makes the BYM hyperparameters directly interpretable. The marginal standard deviation σ_b measures total spatial variation, and the mixing parameter φ measures the proportion attributable to spatial structure. Penalised Complexity priors penalise deviation from the base model (constant risk over all areas), placing probability mass on simpler models unless the data demands complexity.

For the marginal standard deviation, the PC prior is defined by P(σ_b > U) = a. Following the rule of thumb, U = 0.5/0.31 and a = 0.01. For the mixing parameter, P(φ < U) = a with sensible bounds derived from the data structure. These priors have been validated in Bayesian spatio-temporal COVID-19 analysis where BYM2 fitted better than BYM (lower DIC and WAIC) across spatial and temporal structures.

## 10.4 GNN for Urban Spatial Analysis

Graph Neural Networks can capture both the topology of city networks and their spatial context, learning representations that encode why certain intersections become congested or why some neighbourhoods thrive while others struggle. Unlike traditional models treating locations as independent, GNNs understand that a place's character emerges from its connections — making them naturally suited for predicting everything from traffic patterns to inequality risks.

city2graph (Sato 2025) provides a unified pipeline for converting geospatial datasets into GNN-ready graph representations, handling multiple edge types (contiguity, proximity, POI assignment, GTFS transit). For the BYM2 hybrid, the key role of the GNN is to provide spatially-contextualised embeddings that encode road network topology — information unavailable to the purely statistical BYM2 model.

## 10.5 SWMM Dynamic Wave Theory

SWMM's DYNWAVE routing solves the full St Venant equations for unsteady open-channel flow. The governing equations are the continuity equation (∂A/∂t + ∂Q/∂x = q) and momentum equation (∂Q/∂t + ∂(Q²/A)/∂x + gA(∂H/∂x + S_f) = 0). DYNWAVE is the most physically rigorous routing method but requires the Courant–Friedrichs–Lewy (CFL) condition to be satisfied: Δt ≤ Δx/c, where c = √(gA/T) is the wave celerity.

For the Vadodara flood simulation, the CFL constraint with the shortest major road segment (~50m) and wave speed ~3.1 m/s gives a maximum stable timestep of 16 seconds. The 5-second routing step applied in build_swm_fixed.py provides comfortable margin. The original 30-second routing step violated CFL for short segments, forcing DYNWAVE to internally substep to sub-millisecond timesteps — the primary cause of the 17-minute execution.

# 11. Key References

- Anselin, L. (1988). Spatial Econometrics: Methods and Models. Kluwer Academic Publishers.

- Anselin, L., Bera, A.K., Florax, R., Yoon, M.J. (1996). Simple diagnostic tests for spatial dependence. Regional Science and Urban Economics 26(1): 77–104.

- Besag, J., York, J., Mollié, A. (1991). Bayesian image restoration with two applications in spatial statistics. Annals of the Institute of Statistical Mathematics 43(1): 1–20.

- Fotheringham, A.S., Brunsdon, C., Charlton, M. (2002). Geographically Weighted Regression: The Analysis of Spatially Varying Relationships. Wiley.

- Riebler, A., Sørbye, S., Simpson, D., Rue, H. (2016). An intuitive Bayesian spatial model for disease mapping that accounts for scaling. Statistical Methods in Medical Research 25(4): 1145–1165.

- Rue, H., Martino, S., Chopin, N. (2009). Approximate Bayesian inference for latent Gaussian models using integrated nested Laplace approximations. JRSS Series B 71(2): 319–392.

- Sato, Y. (2025). city2graph: Transform geospatial relations into graphs for spatial network analysis and GNNs. doi:10.5281/zenodo.15858845.

- Boeing, G. (2025). Modeling and Analyzing Urban Networks and Amenities with OSMnx. Geographical Analysis 57(4): 567–577.

- Hurvich, C.M., Simonoff, J.S., Tsai, C.L. (1998). Smoothing parameter selection in nonparametric regression using an improved Akaike information criterion. JRSS Series B 60(2): 271–293.